Abstract

Medium Voltage Direct Current (MVDC) power systems are increasingly favored for modern shipboard applications due to their high power density and ability to integrate advanced energy storage. However, the lack of natural zero-crossing points in DC circuits necessitates rapid and precise fault detection to prevent system-wide instability. Current supervised machine learning methods for fault localization often rely on extensive labeled datasets of fault conditions, which are rarely available in operational maritime environments. This thesis proposes a semi-supervised fault detection and localization framework utilizing a Long Short-Term Memory Autoencoder (LSTM-AE) and lightweight Multilayer Perceptron (MLP). By training primarily on "normal" operational data, the model reflects a pragmatic approach to real-world data scarcity. Preliminary results indicate that while the model achieves a classification accuracy of approximately 80\% and a False Alarm Rate (FAR) of 5.6\%, it maintains a recall of 1.0. In the context of safety-critical shipboard infrastructure, this perfect recall ensures that no fault goes undetected, prioritizing system protection over absolute precision. These findings demonstrate that semi-supervised architectures offer a viable, robust alternative for fault management in environments where fault data is inherently limited.